产品名称： LB 5.8G 600mW无线微波图像传收发系统

厂家编号：LB-5.8G-TX600

- LB-5.8G-TX600音视频发射器

- LB-5.8G-TX600音视频发射器是工作在5645-5945MH ISM频段内的FM音视频发射器。本产品采用了高集成芯片，具有以下特性

- 发谢器特性：

* 工作频道频率：

A :

CH1:5740    CH2: 5760   CH3: 5780   CH4: 5800

CH5 : 5820     CH6: 5840   CH7:5860   CH8:5880

B :

CH1:5725   CH2: 5745   CH3:5765   CH4:5785

CH5 : 5805    CH6: 5825   CH7:5845   CH8: 5865

C :

CH1:5733   CH2: 5752    CH3: 5771   CH4: 5790

CH5:  5809   CH6: 5828   CH7: 5847   CH8:5866

D:

CH1: 5705   CH2 :5685   CH3:5665   CH4: 5645

CH5:  5885    CH6: 5905   CH7:5925   CH8:5945

* 5.8G FM音视频同步发射

* 小体积33*22*11 mm，紧凑结构，发射功率大- 采用大规模集成电路，可靠性、灵敏度高

* 工作电压7.4V-16V

* 32个工作通道，DIP开关选择设置

* 使用数字锁相环电路，无温飘现象

* 空中通信距离:>25000米

* 视频信号阻抗: 75Ω

* 天线接头: SMA(外螺内孔)

- 应用范围：航模，无线遥控飞机，飞船，玩具，监视器等。

- 引脚定义：

- 产品参数

| 工作频率 | 工作电压 | 工作通道 | 空中距离 | 视频号阻抗 | 天线接头（内孔） |
| --- | --- | --- | --- | --- | --- |
| 5705-5945 | 7.4V-16V | 32 | >3000 | 75Ω | SMA |

Product Name： LB 5.8G 600mWWireless Transmitting & Receiving System

Factory NO.：LB-5.8G-600

- LBTX-5.8G-600 Audio & Video Transmitter

LB-5.8G-TX600 AV Transmitter is the FM AV Transmitter which works between the frequencies of 5725 to 5945MHz ISM. This product selects high integrated chip and has the following characteristics:

- The Characteristics of the Transmitter

1. Working Frequency: 5725MHz-5945MHz

2. 5.8G FM Audio/Video Synchronization transmission.

3. A small size of 55*28*15mm, compact structure and emission powerfully- the use of large scale integrated circuits ensures the high reliability and sensitivity

4. Working Voltage: 7.4V-16V

5. 32 working channels, DIP switch selectable settings

6. Use of digital phase-locked loop circuit and there is no temperature drift phenomenon.

7. Transmitte Distance:>3000 meters.

8. Built-in Microphone and a compact size.

9. Image signal: NTSC / PAL, 1Vp-p (75 Ohm.)

10. Voice signal: dual channel 2Vp-p (2K Ohm.)

11. Antenna connector: SMA (outer screw hole)

12. Weight: 25g

二. Applications: Model Airplane/Wireless RC Plane, RC Boat and other toys.

- Pin Definition: Picture of Frenquency Control

- Product Parameter

| Working Frequency | Working Voltage | Working Channels | Air Distance | Video Signal Impedance |
| --- | --- | --- | --- | --- |
| 5725-5865 | 7.4V-16V | 32 | >3000 | 75Ω |